ADRV9009 Serial Port (/dev/ttyUSB0),
Linux Terminal & IIO Command-Line Reference

Complete Developer Manual for Serial Console Navigation, iio_info, iio_attr, Memory-Mapped AXI Registers, and Automation Scripts
[English & Persian Technical Documentation - راهنمای جامع پورت سریال و دستورات IIO]

| UART Hardware Device | Silicon Labs / FTDI USB-to-UART Bridge (/dev/ttyUSB0, 115200 8-N-1) |
| --- | --- |
| Embedded Linux OS | Analog Devices Kuiper Linux / PetaLinux on ARM Cortex-A53 / Cortex-A9 |
| IIO Subsystem Stack | Linux Industrial I/O (IIO), libiio v0.24+, iio-tools, devmem2 |
| Document Version / Date | v1.2.0 — October 2026 ¦ Technical Reference for eLynxSDR System Developers |

# 1. Serial Interface Architecture & Terminal Connection (/dev/ttyUSB0)

The ADRV9009 carrier board features an FTDI USB-to-UART bridge that provides a primary debug and control console to the embedded Zynq processor. When plugged into a Linux host PC, the port is typically enumerated as `/dev/ttyUSB0` (or `/dev/ttyUSB1`).

# 1. Verify USB-UART enumeration on Host Linux: $ dmesg ¦ grep -i ttyUSB [ 1245.890] ftdi_sio 1-1.2:1.0: FTDI USB Serial Device converter detected [ 1245.891] usb 1-1.2: FTDI USB Serial Device converter now attached to ttyUSB0 # 2. Grant permissions to access serial port: $ sudo chmod 666 /dev/ttyUSB0 # (Or add user to dialout group: sudo usermod -aG dialout $USER) # 3. Connect via picocom (Recommended): $ picocom -b 115200 /dev/ttyUSB0 # 4. Alternative: Connect via minicom: $ minicom -D /dev/ttyUSB0 -b 115200 -8 -N # 5. Alternative: Connect via screen: $ screen /dev/ttyUSB0 115200

■ SERIAL PORT CONFIGURATION SUMMARY Serial Port Standard Parameters: • Baud Rate: 115200 bps ¦ Data Bits: 8 ¦ Parity: None ¦ Stop Bits: 1 (8-N-1) • Flow Control: None (RTS/CTS disabled, XON/XOFF disabled) • To exit picocom, press Ctrl+A followed by Ctrl+X. To exit screen, press Ctrl+A then K.

# 2. Linux Industrial I/O (IIO) Framework & Topology Discovery (iio_info)

The Linux IIO subsystem abstracts the ADRV9009 transceiver into a set of standard devices, channels, and attributes. The `iio_info` utility scans and prints the entire device hierarchy.

# Scan local IIO context (run inside target board serial console): root@analog:~# iio_info # Scan remote IIO context from Host PC over Ethernet: $ iio_info -u "ip:192.168.1.10" # Query specific device only (adrv9009-phy): $ iio_info -u "ip:192.168.1.10" -d adrv9009-phy

Key IIO Devices in the ADRV9009 Hardware System:

| IIO Device Name | Hardware Subsystem | Primary Function & Attributes |
| --- | --- | --- |
| adrv9009-phy | ADRV9009 Transceiver Core | RF LO tuning (altvoltage0), hardwaregain (attenuation/gain), ENSM state, QEC/LOL cals, temp sensor |
| axi-adrv9009-tx-hpc | FPGA DAC DMA & DDS Engine | Cyclic DMA sample buffer playback, Direct Digital Synthesis (DDS) CW tones, phase/scale/frequency controls |
| axi-adrv9009-rx-hpc | FPGA ADC DMA Core | High-speed baseband IQ capture from RX1 / RX2 channels, buffer streaming into userspace |
| hmc7044 | Clock Distribution IC | Generates DEV_CLK (122.88 MHz) and SYSREF pulses for JESD204B deterministic phase alignment |
| led-count-iio / mwipcore0 | Custom FPGA DRFM Baseband IP | Memory-mapped AXI registers at 0x43C30000: TH0 (amplify), PDW (Doppler en), INchann, THCW (Doppler NCO), DACseles |

# 3. iio_attr Command Reference — Frequency, Gain, ENSM & Calibrations

The `iio_attr` command allows reading and writing device and channel attributes directly from the command line.

## 3.1 RF Carrier Frequency Read & Write Commands

# --- 1. Read Current TX / RX Local Oscillator Carrier Frequency (in Hz) --- $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency 2400000000 # --- 2. Set TX / RX LO Carrier Frequency to 5.8 GHz (5,800,000,000 Hz) --- $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency 5800000000 # --- 3. Set RF Carrier to 915 MHz (915,000,000 Hz) --- $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency 915000000 # --- 4. Read RX Baseband Sampling Rate (in Hz) --- $ iio_attr -d adrv9009-phy -c voltage0 -i sampling_frequency 122880000 # --- 5. Read / Set RF Analog Bandwidth (in Hz) --- $ iio_attr -d adrv9009-phy -c voltage0 -i rf_bandwidth 100000000 $ iio_attr -d adrv9009-phy -c voltage0 -i rf_bandwidth 200000000

## 3.2 Hardware Gain & Attenuation Read & Write Commands

# --- 1. Read TX1 Digital Attenuation (in dB, negative scale) --- $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain -10.000000 dB # --- 2. Set TX1 Attenuation to 0 dB (Maximum Output Power) --- $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain 0.0 # --- 3. Set TX1 Attenuation to 25.5 dB --- $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain -25.5 # --- 4. Read RX1 Gain (in dB) --- $ iio_attr -d adrv9009-phy -c voltage0 -i hardwaregain 20.000000 dB # --- 5. Set RX1 Gain to Maximum (+30 dB) in Manual Gain Control (MGC) --- $ iio_attr -d adrv9009-phy -c voltage0 -i gain_control_mode manual $ iio_attr -d adrv9009-phy -c voltage0 -i hardwaregain 30.0 # --- 6. Set RX1 to Automatic Gain Control (Fast Attack AGC) --- $ iio_attr -d adrv9009-phy -c voltage0 -i gain_control_mode fast_attack

## 3.3 Enable State Machine (ENSM), Calibrations & Temperature Monitoring

# --- 1. Query Current Enable State Machine (ENSM) State --- $ iio_attr -d adrv9009-phy ensm_mode radio_on # --- 2. Force Transceiver into 'sleep' or 'radio_on' State --- $ iio_attr -d adrv9009-phy ensm_mode sleep $ iio_attr -d adrv9009-phy ensm_mode radio_on # --- 3. Enable Internal Real-Time Tracking Calibrations --- $ iio_attr -d adrv9009-phy calibrate_rx_qec_en 1 $ iio_attr -d adrv9009-phy calibrate_tx_qec_en 1 $ iio_attr -d adrv9009-phy calibrate_tx_lol_en 1 $ iio_attr -d adrv9009-phy calibrate_tx_lol_ext_en 1 # --- 4. Read On-Chip Junction Temperature (in millidegrees Celsius) --- $ iio_attr -d adrv9009-phy -c temp0 in_temp0_input 42350 # (42350 m°C = 42.35 °C)

# 4. Direct Digital Synthesis (DDS) & Continuous Wave (CW) Tone Control

The `axi-adrv9009-tx-hpc` FPGA core includes hardware DDS synthesizers allowing direct CW tone generation without consuming CPU cycles or streaming DMA buffers.

# --- 1. Generate 10 MHz CW Tone on TX1 (altvoltage0) at Full Scale (1.0) --- $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o frequency 10000000 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o scale 1.0 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o phase 0 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o raw 1 # --- 2. Generate Two-Tone Intermodulation Signal (10 MHz + 12 MHz on TX1) --- $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o frequency 10000000 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o scale 0.5 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage1 -o frequency 12000000 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage1 -o scale 0.5 # --- 3. Disable DDS Tone Generation (Mute TX) --- $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o raw 0 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage1 -o raw 0

# 5. High-Speed DMA Sample Streaming (iio_writedev & iio_readdev)

For arbitrary waveform playback (Spot noise, LFM chirps, Radar pulses) and continuous baseband capture, use `iio_writedev` and `iio_readdev`:

# --- 1. Stream Arbitrary IQ Waveform to DAC Cyclic Buffer (Playback) --- $ iio_writedev -b 262144 -u "ip:192.168.1.10" axi-adrv9009-tx-hpc < files/spot/spot10mhz_200.txt # --- 2. Capture 1 Million Samples from ADC (RX1 + RX2) to File --- $ iio_readdev -b 65536 -s 1000000 -u "ip:192.168.1.10" axi-adrv9009-rx-hpc > rx_capture.raw # --- 3. Capture Continuous Stream with Non-Blocking Buffer --- $ iio_readdev -b 131072 -u "ip:192.168.1.10" axi-adrv9009-rx-hpc ¦ python3 process_stream.py

# 6. Direct Memory-Mapped (AXI) Register Control (0x43C30000 & devmem2)

The custom FPGA DRFM IP core (`led_count_ip_0` / `MUXDACS`) resides at AXI4-Lite base address `0x43C30000`. Registers can be accessed via `devmem2`, IIO debug channel attributes, or `iio_reg`:

| Register Name | Physical Address | Bit Width | Hardware Functional Description |
| --- | --- | --- | --- |
| REG_AMPLY (TH0) | 0x43C30100 | 16-bit | DRFM amplitude linear attenuation / scale factor (out_count1_th0_raw) |
| REG_VGPO_ENABLE (PDW) | 0x43C30110 | 32-bit | DRFM Doppler modulation enable gate (1 = Enabled, 0 = Disabled) |
| REG_VGPO_OFFSET (INchann) | 0x43C30118 | 32-bit | DRFM initial phase offset word for Doppler NCO |
| REG_VGPO_STEP (THCW) | 0x43C3011C | 32-bit signed | DRFM Doppler phase increment: thcw = round(fd * 2^32 / Fs) |
| REG_DRFM_SELECT (DACseles) | 0x43C30138 | 32-bit | DAC Output Mux: 1 = DRFM radar echo; 0 = DMA sample playback / noise |
| REG_TH1 / REG_TH2 | 0x43C30140 / 44 | 32-bit | Secondary amplitude thresholds for automatic target triggering |

# --- 1. Direct AXI Register Write via devmem2 (Inside Serial Console) --- # Select DRFM Echo Path (DACseles = 1 at 0x43C30138): root@analog:~# devmem2 0x43C30138 w 0x00000001 # Set Doppler Frequency Step for +50 kHz shift at Fs = 122.88 MSPS (THCW = +1747627 at 0x43C3011C): root@analog:~# devmem2 0x43C3011C w 0x001AAAEA # Enable Doppler Modulation (PDW = 1 at 0x43C30110): root@analog:~# devmem2 0x43C30110 w 0x00000001 # --- 2. High-Speed Sysfs Raw Attribute Access (Recommended Pure IIO) --- root@analog:~# echo 1 > /sys/bus/iio/devices/iio:device*/out_count15_dacseles_raw root@analog:~# echo 1747627 > /sys/bus/iio/devices/iio:device*/out_count4_thcw_raw # --- 3. Direct Transceiver SPI Register Read via iio_reg --- # Read Product ID SPI Register (Address 0x000): root@analog:~# iio_reg adrv9009-phy 0x000 0x00

# 7. Automation Scripts — Python (pylibiio & pyserial) & Bash

Below is a complete, standalone Python script using `pylibiio` to automate frequency sweeping and power profiling:

#!/usr/bin/env python3 import iio import time # Connect to ADRV9009 over Gigabit Ethernet ctx = iio.Context('ip:192.168.1.10') phy = ctx.find_device('adrv9009-phy') tx_lo = phy.find_channel('altvoltage0', True) tx1 = phy.find_channel('voltage0', True) print('Starting Automated RF Frequency Sweep (1.0 GHz to 5.0 GHz)...') for freq_hz in range(1000000000, 5000000000 + 1, 200000000): tx_lo.attrs['frequency'].value = str(freq_hz) tx1.attrs['hardwaregain'].value = '-5.0' actual_freq = int(tx_lo.attrs['frequency'].value) temp_mc = int(phy.find_channel('temp0').attrs['in_temp0_input'].value) print(f'Locked at: {actual_freq/1e6:.1f} MHz ¦ Atten: -5.0 dB ¦ Temp: {temp_mc/1e3:.1f} °C') time.sleep(0.1) print('Sweep Completed Successfully.')

راهنمای جامع کار با پورت سریال (/dev/ttyUSB0)،
ترمینال لینوکس و دستورات خط فرمان IIO در تراشه ADRV9009

مرجع کامل تنظیمات کنسول سریال، ابزارهای iio_info و iio_attr، کنترل ثبات‌های حافظه‌ای AXI، بافرهای DMA و اسکریپت‌های اتوماسیون

# ۱. ساختار ارتباط سریال و اتصال به ترمینال برد (ttyUSB0)

برد پردازشی ترنسور ADRV9009 مجهز به مبدل داخلی FTDI USB-to-UART است که با اتصال کابل USB به رایانه میزبان لینوکس، یک پورت سریال مجازی با عنوان `/dev/ttyUSB0` ایجاد می‌کند. از طریق این پورت می‌توان به ترمینال خط فرمان (Bash) لینوکس امبدد مستقر بر روی پردازنده Zynq دسترسی پیدا کرده و کلیه دستورات کنترلی IIO، ثبات‌های حافظه و اسکریپت‌های آزمون را اجرا نمود.

■ خلاصه مشخصات فنی ارتباط سریال تنظیمات استاندارد پورت سریال: • نرخ تبادل داده (Baud Rate): ۱۱۵۲۰۰ بیت بر ثانیه ¦ بیت داده: ۸ ¦ بیت توقف: ۱ ¦ توازن (Parity): ندارد (8-N-1) • نرم‌افزارهای ارتباطی در لینوکس: `picocom -b 115200 /dev/ttyUSB0` یا `minicom` یا `screen`. • جهت خروج از محیط picocom کلیدهای ترکیبی Ctrl+A و سپس Ctrl+X را فشار دهید.

# ۲. مرجع دستورات تنظیم فرکانس، گین و کالیبراسیون با iio_attr

## تنظیم فرکانس نوسان‌ساز محلی (Carrier Frequency)

# خواندن فرکانس فعلی فرستنده و گیرنده بر حسب هرتز: $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency # تنظیم فرکانس روی ۲۴۰۰ مگاهرتز (۲.۴ گیگاهرتز): $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency 2400000000 # تنظیم فرکانس روی ۵.۸ گیگاهرتز: $ iio_attr -d adrv9009-phy -c altvoltage0 -o frequency 5800000000

## کنترل تضعیف دیجیتال فرستنده و گین گیرنده

# خواندن تضعیف کانال فرستنده TX1 (مقیاس دسی‌بل منفی): $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain # تنظیم توان حداکثر فرستنده (تضعیف 0 dB): $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain 0.0 # تنظیم ۲۰ دسی‌بل تضعیف بر روی فرستنده: $ iio_attr -d adrv9009-phy -c voltage0 -o hardwaregain -20.0 # تنظیم گین دستی گیرنده RX1 روی ۲۵ دسی‌بل: $ iio_attr -d adrv9009-phy -c voltage0 -i gain_control_mode manual $ iio_attr -d adrv9009-phy -c voltage0 -i hardwaregain 25.0

## تولید تن تک‌فرکانس سینوسی با سینتی‌سایزر سخت‌افزاری DDS

# تولید تن سینوسی ۱۰ مگاهرتز روی کانال TX1 با دامنه کامل: $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o frequency 10000000 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o scale 1.0 $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o raw 1 # خاموش کردن و قطع سیگنال تن خروجی DDS: $ iio_attr -d axi-adrv9009-tx-hpc -c altvoltage0 -o raw 0

## کنترل ثبات‌های پردازش بیس‌باند DRFM در آدرس حافظه 0x43C30000

# فعال‌سازی مد رادار اکو DRFM در ثبات DACseles (آدرس 0x43C30138): root@analog:~# devmem2 0x43C30138 w 0x00000001 # اعمال استپ فاز داپلر معادل ۵۰+ کیلوهرتز در ثبات THCW (آدرس 0x43C3011C): root@analog:~# devmem2 0x43C3011C w 0x001AAAEA # فعال‌سازی مدولاسیون شیفت داپلر در ثبات PDW (آدرس 0x43C30110): root@analog:~# devmem2 0x43C30110 w 0x00000001 # ارسال تضعیف دیجیتال TH0 از طریق Sysfs با سرعت بالا: root@analog:~# echo 1000 > /sys/bus/iio/devices/iio:device*/out_count1_th0_raw